CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Henry	Mesa

Student #: 1008099273

Student Email: mesah1@unlv.nevada.edu

Primary Github address: https://github.com/mesah1

Directory: https://github.com/mesah1/submissions

COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Xplained-Mini

Multifunction-Shield

Block diagram with pins used in the Atmega328PB

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DesignAssignment3A.c

*

* Created: 10/16/2019 12:59:50 PM

* Author : Henry Mesa

*/

#define F_CPU 16000000UL

#define UBRR_9600 103

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

//Function declaration

void read_adc(void);

void adc_init(void);

void USART_init( );

void USART_tx_string(char *data);

volatile float adc;

char ar[20];

int main(void)

{

adc_init();						//intialize ADC

USART_init(UBRR_9600);			//initialize USART

USART_tx_string("Hello there!\n");

_delay_ms(500);

while (1)

{

read_adc();

snprintf(ar, sizeof(ar),"%f\r\n",adc);	//print

USART_tx_string("resistance = ");

USART_tx_string(ar);

_delay_ms(1000);				//wait 1s

}

}

/* INITIALIZING ADC */

void adc_init(void){

//ADC ENABLE AND SET-UP

ADMUX = (0 << REFS1)|    // external cap at AREF

(1 << REFS0)|    // AVcc -  Reference Selection INTERNAL

(0 << ADLAR)|    // ADC Left Adjust Result

(0 << MUX2)|     // Analog Channel Selection Bits

(0 << MUX1)|     // ADC0 (PC0 PIN23)

(0 << MUX0);

ADCSRA = (1 << ADEN)|    // ADC ENable

(0 << ADSC)|     // ADC Start Conversion

(0 << ADATE)|    // ADC Auto Trigger Enable

(0 << ADIF)|     // ADC Interrupt Flag

(0 << ADIE)|     // ADC Interrupt Enable

(1 << ADPS2)|    // ADC Prescaler Select Bits

(0 << ADPS1)|

(1 << ADPS0);

}

/* READ ADC PINS */

void read_adc(void){

unsigned char i = 4;

adc = 0;

while (i--)

{

ADCSRA |= (1 << ADSC);

while(ADCSRA & (1 << ADSC));

adc+= ADC;

_delay_ms(50);

}

adc = adc / 4;  // Average a few samples

}

/* INITIALIZING USART (RS-232) */

void USART_init(unsigned int ubrr){

UBRR0H = (unsigned char)(ubrr >> 8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0); //transmitter enabled

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00); //5-bit characters

}

/* SEND STRING TO RS-232 */

void USART_tx_string(char *data){

while ((*data != '\0')){

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = *data;

data++;

}

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/uERYIGaK3jE

- GITHUB LINK OF THIS DA

https://github.com/mesah1/submissions/tree/master/DA3A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Henry Mesa