CPE301 – SPRING 2019

# Design Assignment 4A

Student Name: Henry	Mesa

Student #: 1008099273

Student Email: mesah1@unlv.nevada.edu

Primary Github address: https://github.com/mesah1

Directory: https://github.com/mesah1/submissions

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

ATmega328PB

Multifunction Shield

Breadboard

DC Motor

MD08A

Block diagram with pins used in the Atmega328PB:

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* Assignment4A.c

*

* Created: 11/4/2019 8:01:41 AM

* Author: Henry Mesa

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

/*	When switch (SW) goes LOW motor will stop

SW is connected to PC1 always HIGH until SW is

pressed	*/

#define SW (PINC |= (1 << 1))

void read_adc(void);

void adc_init(void);

volatile float adc;

int main( ){

adc_init();

PORTC |= 0b00000011; //pull-up enable PC0 & PC1

DDRB |= (1 << DDB1);    // PB1 is an output

ICR1 = 0xFFFF;	//set TOP to 16bit

TCCR1A |= (1 << COM1A1)|(1 << COM1B1);    // set to non-inverting mode

TCCR1A |= (1 << WGM11);

TCCR1B |= (1 << WGM12)|(1 << WGM13);    // set Fast PWM mode using ICR1 as TOP

TCCR1B |= (1 << CS10);    //no prescaler

while (SW)

{

read_adc();		//call function

_delay_ms(50);

if ((adc >= 62258) && (adc < 65535 ))	//keeping pwm at 95% max

{

OCR1A = 62258;

_delay_ms(20);

}

else if ((adc < 62257) && (adc >= 3277)) //increasing pwm as resistor

{						//value increases

OCR1A = adc + 30000;

_delay_ms(20);

}

else

OCR1A = 0x00;	//if resistor value is less than 5% then pwm is 0%

}

}

/* INITIALIZING ADC */

void adc_init(void){

//ADC ENABLE AND SET-UP

ADMUX = (0 << REFS1)|    // external cap at AREF

(1 << REFS0)|    // AVcc -  Reference Selection INTERNAL

(1 << ADLAR)|    // ADC Left Adjust Result

(0 << MUX2)|     // Analog Channel Selection Bits

(0 << MUX1)|     // ADC0 (PC0 PIN23)

(0 << MUX0);

ADCSRA = (1 << ADEN)|    // ADC ENable

(0 << ADSC)|     // ADC Start Conversion

(0 << ADATE)|    // ADC Auto Trigger Enable

(0 << ADIF)|     // ADC Interrupt Flag

(0 << ADIE)|     // ADC Interrupt Enable

(1 << ADPS2)|    // ADC Prescaler Select Bits 128

(0 << ADPS1)|

(1 << ADPS0);

}

/* READ ADC PINS */

void read_adc(void){

unsigned char	 i = 4;

adc = 0;

while (i--)

{

ADCSRA |= (1 << ADSC);

while(ADCSRA & (1 << ADSC));

adc+= ADC;

_delay_ms(50);

}

adc = adc / 8;  // Average a few samples

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/jgq1-BrRsHk

- GITHUB LINK OF THIS DA

https://github.com/mesah1/submissions/tree/master/DA4A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Henry Mesa