CPE301 – SPRING 2019

# Design Assignment 4B

Student Name: Henry	Mesa

Student #: 1008099273

Student Email: mesah1@unlv.nevada.edu

Primary Github address: https://github.com/mesah1

Directory: https://github.com/mesah1/submissions

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

ATmega328pb

Multi-Function Shield

Analog Servo 3001HB

Stepper Motor 28BYJ-48

MD08A

ULN2003AN

Power Supply

Breadboard

Wires

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA4B.c

*

* Created: 11/11/2019 9:23:33 AM

* Author : Henry Mesa

*	THIS PROGRAM WILL CONTROL THE VELOCITY OF A STEPPER MOTOR

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

//Functions to manipulate ADC input

void read_adc(void);

void adc_init(void);

//ADC storage

volatile float adc;

int main(void)

{

adc_init(); //Initializing ADC

//Configuring input and output pins

PORTC |= 0b00000001; //input pull-up enable PC0

DDRD = 0xFF; DDRB = 0xFF;//outputs

//Configuration of Timer1

TCCR1A |= (1 << COM1A1) |(1 << COM1B1) |(1 << WGM11); //non-inverted PWM

//PRESCALER = 64

TCCR1B |= (1 << WGM13) |(1 << WGM12) |(1 << CS11) |(1 << CS10);

// freq = 50Hz  Period = 20ms

ICR1 = 4999;

while (1)

{

read_adc();		//Read ADC value

_delay_ms(25);

while (1)

{

//Stepper Motor Instructions

PORTD = 0b01100110;

_delay_ms(50);

PORTD = 0b11001100;

_delay_ms(50);

PORTD = 0b10011001;

_delay_ms(50);

PORTD = 0b00110011;

_delay_ms(50);

if ((adc >= 973) && (adc < 1024 ))	//when adc value is at its max

{					//delay the least

_delay_ms(10);

}

else if ((adc < 972) && (adc >= 768)) //decreasing pwm as resistor

{					   //value decreases

_delay_ms(50);

}

else if ((adc < 767) && (adc >= 51)) //decreasing pwm as resistor

{					   //value decreases

_delay_ms(100);

}

else

{

PORTD = 0b00000000;//if resistor value is less than 5% then pwm is 0%

_delay_ms(100);

}

}

}

}

/* INITIALIZING ADC */

void adc_init(void){

//ADC ENABLE AND SET-UP

ADMUX = (0 << REFS1)|    // external cap at AREF

(1 << REFS0)|    // AVcc -  Reference Selection INTERNAL

(1 << ADLAR)|    // ADC Left Adjust Result

(0 << MUX2)|     // Analog Channel Selection Bits

(0 << MUX1)|     // ADC0 (PC0 PIN23)

(0 << MUX0);

ADCSRA = (1 << ADEN)|    // ADC ENable

(0 << ADSC)|     // ADC Start Conversion

(0 << ADATE)|    // ADC Auto Trigger Enable

(0 << ADIF)|     // ADC Interrupt Flag

(0 << ADIE)|     // ADC Interrupt Enable

(1 << ADPS2)|    // ADC Prescaler Select Bits 128

(0 << ADPS1)|

(1 << ADPS0);

}

/* READ ADC PINS */

void read_adc(void){

unsigned char	 i = 4;

adc = 0;

while (i--)

{

ADCSRA |= (1 << ADSC);

while(ADCSRA & (1 << ADSC));

adc+= ADC;

_delay_ms(50);

}

adc = adc / 8;  // Average a few samples

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

/*

* DA4B.c

*

* Created: 11/11/2019 9:23:33 AM

* Author : Henry

* 	THIS PROGRAM CONTROLS DIRECTION OF MOVEMENT OF A SERVO MOTOR

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

//Functions to manipulate ADC input

void read_adc(void);

void adc_init(void);

//ADC storage

volatile float adc;

int main(void)

{

adc_init(); //Initializing ADC

//Configuring input and output pins

PORTC |= 0b00000001; //input pull-up enable PC0

DDRD = 0xFF; DDRB = 0xFF;//outputs

//Configuration of Timer1

TCCR1A |= (1 << COM1A1) |(1 << COM1B1) |(1 << WGM11); //non-inverted PWM

//PRESCALER = 64

TCCR1B |= (1 << WGM13) |(1 << WGM12) |(1 << CS11) |(1 << CS10);

// freq = 50Hz  Period = 20ms

ICR1 = 4999;

while (1)

{

read_adc();		//Read ADC value

_delay_ms(25);

//Servo Motor Command

OCR1A = adc;

_delay_ms(250);

}

}

/* INITIALIZING ADC */

void adc_init(void){

//ADC ENABLE AND SET-UP

ADMUX = (0 << REFS1)|    // external cap at AREF

(1 << REFS0)|    // AVcc -  Reference Selection INTERNAL

(1 << ADLAR)|    // ADC Left Adjust Result

(0 << MUX2)|     // Analog Channel Selection Bits

(0 << MUX1)|     // ADC0 (PC0 PIN23)

(0 << MUX0);

ADCSRA = (1 << ADEN)|    // ADC ENable

(0 << ADSC)|     // ADC Start Conversion

(0 << ADATE)|    // ADC Auto Trigger Enable

(0 << ADIF)|     // ADC Interrupt Flag

(0 << ADIE)|     // ADC Interrupt Enable

(1 << ADPS2)|    // ADC Prescaler Select Bits 128

(0 << ADPS1)|

(1 << ADPS0);

}

/* READ ADC PINS */

void read_adc(void){

unsigned char	 i = 4;

adc = 0;

while (i--)

{

ADCSRA |= (1 << ADSC);

while(ADCSRA & (1 << ADSC));

adc+= ADC;

_delay_ms(50);

}

adc = adc / 8;  // Average a few samples

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/4UEZMuFaITc

- GITHUB LINK OF THIS DA

https://github.com/mesah1/submissions/tree/master/DA4B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Henry Mesa