CPE301 – SPRING 2019

# MIDTERM 1

Student Name: Henry	Mesa

Student #: 1008099273

Student Email: mesah1@unlv.nevada.edu

Primary Github address: https://github.com/mesah1

Directory: https://github.com/mesah1/submissions

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

Atmega328P, LM35, ESP01

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* Midterm.c

*

* Created: 10/26/2019 2:06:31 PM

* Author : Henry Mesa

* TOOL: LM35, EXPLAINED-MINI(ATMEGA328PB), ESP01

* THIS CODE WILL UPLOAD TEMPERATURE VALUES TO THINGSPEAK.COM THROUGHT USART

*

*/

#define F_CPU 16000000UL

#define UBRR_9600 103

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <stdlib.h>

#include <avr/interrupt.h>

//Function declaration

void read_adc(void);

void adc_init(void);

void USART_init( );

void USART_tx_string(char *data);

volatile float adc;

char ar[255];

int main(void)

{

adc_init();					//intialize ADC

read_adc();

USART_init(UBRR_9600);			//initialize USART

USART_tx_string("Connected!\n");

snprintf(ar, sizeof(ar),"%f\n\r",adc/128);	//print

USART_tx_string(ar);

while (1)

{

}

}

/* INITIALIZING ADC */

void adc_init(void){

//ADC ENABLE AND SET-UP

ADMUX = (0 << REFS1)|    // external cap at AREF

(1 << REFS0)|    // AVcc -  Reference Selection INTERNAL

(1 << ADLAR)|    // ADC Left Adjust Result

(1 << MUX2)|     // Analog Channel Selection Bits

(0 << MUX1)|     // ADC0 (PC5 PIN28)

(1 << MUX0);

ADCSRA = (1 << ADEN)|    // ADC ENable

(0 << ADSC)|     // ADC Start Conversion

(0 << ADATE)|    // ADC Auto Trigger Enable

(0 << ADIF)|     // ADC Interrupt Flag

(0 << ADIE)|     // ADC Interrupt Enable

(1 << ADPS2)|    // ADC Prescaler

(0 << ADPS1)|

(1 << ADPS0);

// Timer/Counter1 Interrupt Mask Register

TIMSK1 |= (1<<TOIE1); // enable interrupt flag

// Set Prescalar

TCCR1B = 5; // setting the prescalar to 1024

// Set timer

TCNT1 = 7000; // set TCNT1

_delay_ms(1000); // wait a bit

sei(); //interrupt

}

/* READ ADC PINS */

void read_adc(void){

unsigned char i = 4;

adc = 0;

while (i--)

{

ADCSRA |= (1 << ADSC);

while(ADCSRA & (1 << ADSC));

adc+= ADC;

_delay_ms(50);

}

adc = adc / 4;  // Average a few samples

}

/* INITIALIZING USART (RS-232) */

void USART_init(unsigned int ubrr){

UBRR0H = (unsigned char)(ubrr >> 8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0)|(1 << RXEN0)| ( 1 << RXCIE0); //transreciver enabled and RX interrupt

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00); //5-bit characters

}

/* SEND STRING TO RS-232 */

void USART_tx_string(char *data){

while ((*data != '\0')){

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = *data;

data++;

}

}

void UART_sendString(volatile unsigned char AT[]){

volatile unsigned char len = 0;

volatile unsigned char i;

while (AT[len] != 0){

len++;

}

for (i = 0x00; i < len; i++)

{

//Wait for the transmitter to finish

while (!(UCSR0A & (1 << UDRE0)))

{

UDR0 = AT[i];

}

}

}

ISR(TIMER1_OVF_vect){

unsigned char AT_COMMMANDS[] = "AT\n\r"; //AT Commands

unsigned char CWMODE[] ="AT+CWMODE=1\n\r"; //Set the mode

unsigned char CWJAP[] ="AT+CWJAP=\"AquaMan\",\"AMI@6280#life\"\r\n"; // WIFI username and password

unsigned char CIPMUX[] ="AT+CIPMUX=0\n\r";

unsigned char CIPSTART[] ="AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\n\r";

unsigned char CIPSEND[] ="AT+CIPSEND=51\n\r";

unsigned char DATA[]="GET  /update?api_key=0DSHG7MOCFW2VGFI&field1=";

unsigned char CLOSE[]="AT+CIPCLOSE\n\r";

read_adc();

snprintf(ar, sizeof(ar),"%f\n\r",adc/128);	//print

UART_sendString(AT_COMMMANDS);

_delay_ms(3000);

UART_sendString(CWMODE);

_delay_ms(2000);

UART_sendString(CWJAP);

_delay_ms(3000);

UART_sendString(CIPMUX);

_delay_ms(2000);

UART_sendString(CIPSTART);

_delay_ms(3000);

UART_sendString(CIPSEND);

_delay_ms(3000);

UART_sendString(DATA);

UART_sendString(ar);

_delay_ms(3000);

UART_sendString(CLOSE);

_delay_ms(2000);

USART_tx_string(ar);

TCNT1 = 7000; //resets timer

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/xVnoOiG9VTE

- THINGSPEAK LINK

https://thingspeak.com/channels/895725

- GITHUB LINK OF THIS DA

https://github.com/mesah1/submissions/tree/master/Midterm

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

HENRY MESA